CPE301 – SPRING 2019

# Design Assignment 5

Student Name: Tyler Gardenhire

Student #: 8000450294

Student Email: gardenhi@unlv.nevada.edu

Primary Github address: gardenhi@unlv.nevada.edu

Directory: https://github.com/tylergardenhire/submission_projects.git

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7 w/ AVR assembly and simulator, Atmega328p board.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Task 1 C code:

//set clock frequency

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdio.h>

#include <string.h>

//	Set up UART for printf();

#define BAUD 9600

#include "inc\STDIO_UART.c"

//	Include nRF24L01+ library

#include "inc\nrf24l01.c"

#include "inc\nrf24l01-mnemonics.h"

#include "inc\spi.c"

void print_config(void);

//	Used in IRQ ISR

volatile bool message_received = false;

volatile bool status = false;

void ADC_init (void);

volatile unsigned char ADCtemp[5];

volatile uint8_t ADCvalue;

int main(void)

{

//set message to send

char tx_message[32];				//define string array

strcpy(tx_message,"It's working!");	//copy string into array

uart_init(); //initialize UART

ADC_init(); //initialize ADC

nrf24_init(); //initialize nRF24L01+

print_config();

nrf24_start_listening(); //listen to incoming messages

strcpy(tx_message,"Initializing Chat Room...");	//copy string into array

nrf24_send_message(tx_message);

while (1)

{

if (message_received)

{

//print message

message_received = false;

printf("Received Temperature: %s\n\n",nrf24_read_message());

//	Send message as response

_delay_ms(500);

status = nrf24_send_message(ADCtemp);

if (status == true) printf("Temperature Successfully Sent\n\n");

}

}

}

//interrupt on IRQ pin

ISR(INT0_vect)

{

message_received = true;

}

//interrupt for Temperature Sensor

ISR(ADC_vect)

{

volatile unsigned int j=0;

char temp[5];

ADCvalue = (ADCH << 1);		 //shifts the value left to one place

itoa(ADCvalue, temp, 10);	 //converts integers to string

while (j<5)					 //transfers the temp string from itoa() to ADCtemp

{

ADCtemp[j] = temp[j];

j++;

}

}

//prints configuration

void print_config(void)

{

uint8_t data;

printf("Startup successful\n\n nRF24L01+ configured as:\n");

printf("-------------------------------------------\n");

nrf24_read(CONFIG,&data,1);

printf("CONFIG		0x%x\n",data);

nrf24_read(EN_AA,&data,1);

printf("EN_AA			0x%x\n",data);

nrf24_read(EN_RXADDR,&data,1);

printf("EN_RXADDR		0x%x\n",data);

nrf24_read(SETUP_RETR,&data,1);

printf("SETUP_RETR		0x%x\n",data);

nrf24_read(RF_CH,&data,1);

printf("RF_CH			0x%x\n",data);

nrf24_read(RF_SETUP,&data,1);

printf("RF_SETUP		0x%x\n",data);

nrf24_read(STATUS,&data,1);

printf("STATUS		0x%x\n",data);

nrf24_read(FEATURE,&data,1);

printf("FEATURE		0x%x\n",data);

printf("-------------------------------------------\n\n");

}

void ADC_init (void)

{

//ADC Multiplexer Selection Register

ADMUX = (1 << REFS0) |		 //voltage reference during conversion, "AVcc with external capacitor at AREF pin"

(1 << ADLAR);				 //left adjust ADC conversion result in ADC Data Register

//ADC Control and Status Register A

ADCSRA = (1 << ADEN) |		 // ADC enable

(1 << ADSC) |		 //ADC Start Conversion

(1 << ADATE) |		 //ADC Auto Trigger enable

(1 << ADIE) |		 //ADC Interrupt enable

(1 << ADPS2) |

(1 << ADPS1) |

(1 << ADPS0);		 //ADPS2:0 = 111 = 128 prescaler

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

N/A

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

N/A

- GITHUB LINK OF THIS DA

https://github.com/tylergardenhire/submission_projects.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

TYLER GARDENHIRE