CPE301 – SPRING 2019

# Midterm 2

Student Name: Tyler Gardenhire

Student #: 8000450294

Student Email: gardenhi@unlv.nevada.edu

Primary Github address: gardenhi@unlv.nevada.edu

Directory: https://github.com/tylergardenhire/submission_projects.git

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7 w/ AVR assembly, Atmega328p board, FTDI chip, and ESP module used.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* Midterm 2

* Created: 5/13/2019 3:21:23 PM

* Author : Tyler Gardenhire

*/

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <stdlib.h>

#include <stdint.h>

#include "SparkFun_APDS9960.h"

#include "i2c_master.h"

#define F_CPU 16000000UL

#define BAUD 9600

#define FOSC 16000000

#define UBRREQ FOSC/16/BAUD -1

#define APDS9960_WRITE 0x72

#define APDS9960_READ 0x73

void UART_init (void);

void APDS_init (void);

int uart_putchar( char c, FILE *stream);

FILE str_uart = FDEV_SETUP_STREAM(uart_putchar, NULL , _FDEV_SETUP_WRITE);

void getreading(void);

uint16_t red;

uint16_t green;

uint16_t blue;

int main(void)

{

UART_init(); //initialize uart values

APDS_init(); //initialize APDS9960

i2c_init();  //initialize I2C

stdout = &str_uart;

red = 0;

green = 0;

blue = 0;

_delay_ms(5000);

printf("AT+CWMODE=3\r\n"); 	//set AP’s info

_delay_ms(5000);

printf("AT+CWJAP=\"eos2f8\",\"122gfh48\"\r\n"); //connect to Internet

while(1)//send values through the cloud

{

_delay_ms(5000);

printf("AT+CIPMUX=0\r\n"); 	//enable single connection

_delay_ms(5000);

printf("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n"); //start the connection to the cloud

//set length of data to be sent

_delay_ms(5000);

getreading();

printf("AT+CIPSEND=104\r\n");

printf("GET https://api.thingspeak.com/update?api_key=thingspeakkey&field1=0%05u&field2=%05u&field3=%05u\r\n", red, green, blue);

_delay_ms(3000);

}

}

void getreading()

{

uint8_t redH, redL;

uint8_t greenH, greenL;

uint8_t blueH, blueL;

//red

i2c_readReg(APDS9960_WRITE, APDS9960_RDATAH, &redH, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_RDATAL, &redL, 1);

//green

i2c_readReg(APDS9960_WRITE, APDS9960_GDATAH, &greenH, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_GDATAL, &greenL, 1);

//blue

i2c_readReg(APDS9960_WRITE, APDS9960_BDATAH, &blueH, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_BDATAL, &blueL, 1);

red = (redH << 8) | redL;

green = (greenH << 8) | greenL;

blue = (blueH << 8) | blueL;

// THRESHOLD

if (red > 255)

red = 255;

if (green > 255)

green = 255;

if (blue > 255)

blue = 255;

}

void APDS_init(void)

{

uint8_t setup;

i2c_readReg(APDS9960_WRITE, APDS9960_ID, &setup,1);

if(setup != APDS9960_ID_1) while(1);

setup = 1 << 1 | 1<<0 | 1<<3 | 1<<4;

i2c_writeReg(APDS9960_WRITE, APDS9960_ENABLE, &setup, 1);

setup = DEFAULT_ATIME;

i2c_writeReg(APDS9960_WRITE, APDS9960_ATIME, &setup, 1);

setup = DEFAULT_WTIME;

i2c_writeReg(APDS9960_WRITE, APDS9960_WTIME, &setup, 1);

setup = DEFAULT_PROX_PPULSE;

i2c_writeReg(APDS9960_WRITE, APDS9960_PPULSE, &setup, 1);

setup = DEFAULT_POFFSET_UR;

i2c_writeReg(APDS9960_WRITE, APDS9960_POFFSET_UR, &setup, 1);

setup = DEFAULT_POFFSET_DL;

i2c_writeReg(APDS9960_WRITE, APDS9960_POFFSET_DL, &setup, 1);

setup = DEFAULT_CONFIG1;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG1, &setup, 1);

setup = DEFAULT_PERS;

i2c_writeReg(APDS9960_WRITE, APDS9960_PERS, &setup, 1);

setup = DEFAULT_CONFIG2;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG2, &setup, 1);

setup = DEFAULT_CONFIG3;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG3, &setup, 1);

}

void USART_putstring(char *StringPtr)

{

while ((*StringPtr != '\0'))

{

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = *StringPtr; //UDR0 register grabs the value given from the parameter

StringPtr++;

}

}

void UART_init(void)

{

uint16_t baud_rate = UBRREQ;

UBRR0H = baud_rate >> 8;

UBRR0L = baud_rate & 0xFF;

UCSR0B = ( 1 <<RXEN0)|( 1 <<TXEN0);	//enable receiver and transmitter

UCSR0C = (3 <<UCSZ00); 	// Set frame format: 8data, 1stop bit

}

int uart_putchar(char c, FILE *stream)

{

while ( !( UCSR0A & ( 1 <<UDRE0)) ); //wait until buffer is empty

UDR0 = c; //data goes into buffer

return 0;

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

N/A

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/J3iwdaao_Ws

- GITHUB LINK OF THIS DA

https://github.com/tylergardenhire/submission_projects.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

TYLER GARDENHIRE